Equipment: Freeze Dryer for Fruit Processing

1. Production Requirement

•	Daily Capacity: 500 kg/day of fruit

•	Corresponding Freeze Dryer Size: approx. 50㎡ or 30 m2 please adv

2. Process Conditions

•	Freezing Temperature: –40°C

•	Primary Drying (under vacuum): –20°C to 0°C

•	Secondary Drying: +20°C to +40°C

•	Vacuum Requirement: 15–25 Pa

3. Power Supply

•	Voltage: 400V, 50Hz, 3-phase (Mauritius standard)

4. Heating & Cooling

•	Heating Method: Steam heating

•	Supporting Equipment Required: Steam generator and related auxiliary systems

•	Cooling: Standard water cooling (please specify configuration in quotation)

5. Material & Construction

•	Product Contact Parts: Stainless Steel 304 (food-grade)

•	External Parts: Supplier to specify standard configuration

•	Compliance: Food-grade design (CE/GMP not required, but optional offer can be provided)

6. Equipment Layout & Configuration

•	Shelf/Tray Area: approx. 50㎡

•	Shelf Temperature Range: –40°C to +40°C

•	Tray/Shelf Design: Supplier to propose

•	Vacuum Pump System: Included (please specify type and brand in quotation)

7. Supporting Systems

Please include options for the following in your proposal:

•	Steam generator (for heating)

•	Chiller/cooling water system

•	Air compressor (if required for valve actuation)

•	Control system (preferably PLC + touch screen, remote monitoring optional)

8. Quotation Requirements

Please provide a detailed quotation including:

•	Technical proposal with equipment specifications

•	Main component brands (electrical/automation, vacuum pump, refrigeration system)

•	Price breakdown (main equipment + supporting systems)

•	Delivery time (ex-works)

•	Warranty period

•	After-sales service scope (installation, commissioning, training)

9. Destination & Packaging

•	Installation Location: Mauritius

•	Packaging: Standard export wooden case (fumigated)